SAFETY CHECKLIST

(you are NOT officially registered until this is signed by the Safety Officer

and YOU turn it back in to registration)

Sailor _____________________ Sail Number ________Safety Officer_________Pass__ Fail__

BELOW ARE MINIMUM SUGGESTED SAFETY REQUIREMENTS

SAILOR:

- Water bottle

- Recommended: sunscreen, hat, polarized sunglasses and a smile!

LIFE JACKET:

- Must be US Coast Guard Approved, proper size and snug fit. No rips, tears, worn or threadbare fabric. Good to excellent condition.

- Safety whistle. Sailor to demonstrate loudness.

BOAT:

- Flotation bags… three “fiber reinforced” external or one “fiber reinforced” in the stern and two of any type internal bags in side tanks.

- External flotation bags must have three straps secured to hull. (Pre-IOD95 and wooden Optis may substitute a piece of line tied between the outer two straps in lieu of a center strap.)

- Mast tie-in system. Any positive means of securing mast into boat in a capsize situation, usually a line secured to the thwart. Locking collar acceptable.

- Bailer, at least half gallon size, durable, with lanyard.

- Recommended: two rigid 1 gallon bailers w/ lanyards.

- Painter, positive flotation, at least 1/4” (5mm) diameter, 28’ length, ABSOLUTELY NO KNOTS except for attachment to mast step and bowline (of about 6” diameter) in running end. Must NOT be lead through bow drain hole (causes chafe).

- Skipper should demonstrate proper, safe stowage of painter. (coiled, no locking hitches, stuffed under bow rail).

- Daggerboard secured to boat with a lanyard.

- Praddel or paddle  (USODA has not been enforcing this, but it is a strongly advised safety item!).

RIGGING:

- Mast must have signs of positive flotation, ie- plugs in each end, or plug in bottom and cork in top. All 3 spars should float.

- Boom bridle less than 10cm (4”) at widest point from boom.

- Boom bridle preventer recommended.

- Bridle/mainsheet connection. Must be a shackle-type device capable of easy disconnect (screw pins or tying with line is not acceptable). MUST NOT BE SELF OPENING OR ABLE TO SNAG CLOTHING, LIFEJACKET OR TOWLINE. Stopper knot in end of sheet recommended.

SAILS:

- Unique sail number, clearly legible from committee boat. Legal size and spacing.